# Enterprise NOC Simulation - Topology Details

**Author:** Salmata Lamin

**Environment:** Cisco Packet Tracer

**Objective:** Simulate an enterprise NOC network with VLAN segmentation, inter-VLAN routing, and server integration.

## 🧩 Device Inventory

| Device | Role | IP Address | VLAN | Subnet | Notes |

|---------|------|-------------|-------|---------|-------|

| **CORE-RTR01** | Core Router | 10.10.10.1 / 10.10.20.1 / 10.10.30.1 | - | Multiple sub-interfaces | Handles inter-VLAN routing |

| **SW01** | Main Switch | N/A | VLAN 10 & 20 | Trunk to CORE-RTR01 | Connects Admin & Ops VLANs |

| **SW02** | Secondary Switch | N/A | VLAN 30 | Trunk to CORE-RTR01 | Connects Security VLAN |

| **NOC-SRV01** | Server | 10.10.10.10 | VLAN 10 | 10.10.10.0/24 | Central file/log server |

| **PC-ADMIN** | Admin Workstation | 10.10.10.24 | VLAN 10 | 10.10.10.0/24 | Management access |

| **PC-OPS** | Operations Workstation | 10.10.20.24 | VLAN 20 | 10.10.20.0/24 | NOC monitoring |

| **PC-SEC** | Security Workstation | 10.10.30.24 | VLAN 30 | 10.10.30.0/24 | SOC/Security testing |

---

## 🌐 VLAN Summary

| VLAN ID | Department | Subnet | Gateway |

|----------|-------------|---------|----------|

| 10 | Admin | 10.10.10.0/24 | 10.10.10.1 |

| 20 | Operations | 10.10.20.0/24 | 10.10.20.1 |

| 30 | Security | 10.10.30.0/24 | 10.10.30.1 |

---

## 🔗 Connections

| Connection | Interface | Description |

|-------------|------------|--------------|

| CORE-RTR01 ↔ SW01 | G0/0.1–0.3 | Trunk for VLANs 10, 20, 30 |

| SW01 ↔ SW02 | G0/1 | Trunk link |

| PCs ↔ Switches | F0/x | Access ports for VLANs 10, 20, and 30 |

---

## ✅ Verification Commands

Use these to confirm successful setup:

```bash

show vlan brief

show ip interface brief

show ip route

ping 10.10.10.24

ping 10.10.20.24

ping 10.10.30.24